The SMRT Network:  An introductory overview.

The Active Network infrastructure

We use a set of Cisco 3750 layer 3 switches at the core of the LAN. There are 6 devices linked at the backplanes to form one logical unit for resiliency. The edge devices are all managed HP devices. The LAN spans all three buildings – linked via fibre back to the main core.

There are several vLANs on the network - including: for the doors (Salto), for the kitchen (Cunninghams), for wireless devices, one for projectors, and another for the switches themselves. We access the internet via the LA - and run IP address ranges that they have supplied. Only subnets of these ranges will be allowed through their firewall. All IP records are listed in the 'staticIP.xls' sheet. See the Network Config section of the wiki.

The virtualisation hardware

We have 2 IBM 1U servers now upgraded to house 2 CPUs and 72Gb of RAM each. These disk-less servers boot vmware ESXi from USB devices, and connect to 3 NetApp SANs for storage. The NetApp SANs offer deduplicated storage on FAS01 (2TB - SAS), FAS02 (7TB SATA) and FAS03 (2 TB newer SAS). There are paired 1GB links between the servers and each SAN.

Backup

There is a Dell Windows server 2012 server in the 6FC running Veeam 7. The Veeam system pulls the VMs off to its internal 20TB of storage.

The wireless

Wireless is provided by an adaptive Aruba 3400 controller and some 62 access points. The system intelligently manages the channels and, to a certain extent, the hosts connecting to it. We currently run wireless a, g and n.

The servers

We have some 25 VMs running windows servers from 2003, 2008R2 and some linux VMs. There is a single physical Mac server connecting the 30 Macs on the network to the same domain.

The services

The users of the system have access to a variety of desktop and laptop domain computers, all running windows 7 and offering a variety of services including: File & print (printing is monitored by Papercut), SIMS, Internet & email, and Office 2010. All teaching spaces have a consistent AV experience provided by a docking station connected to a projector, and sound. Several faculties have Interactive white boards in addition to this. All students and teaching staff have accounts on our VLE; we are transitioning our VLE from Fronter to Moodle. There are a variety of subject specific software packages available.

We also support the kitchen’s cashless catering system (Cunninghams) as well as the Electronic door locking system (Salto) and Building Management System (Desigo)

There are a number of back end services to deliver the above including the networking and wireless (detailed above) and the backup system in the 6FC (6th form centre) running Veeam on Windows 2012.

The monitoring

We have network monitoring tools in the form of a Cacti server for longer historical benchmark reference points, also offering Syslog services. We also have Spiceworks installed, as well as VMWare vSphere monitoring, and a NetApp storage monitoring system.

The Team

The team consists of a network manager and 3 technicians who manage the network and support services. They are: Al, Darren, Mark, & Matt. There is also an excellent eLearning technician (Nick) who manages the website, our VLE and our Clickview video playback system.

We work closely with the premises team to support the building management system, and door locking system – the Site Premises Manager is John Shackell.