NON-CONVERGED TOPOLOGY (DCB PFC/ETS)                                        OS10, 10Gbe/100Gbe

QLOGICTCP RDMA	QUICK-START WSSD

| S4112F-ON (TOR1) | S4112F-ON (TOR2) |
| --- | --- |
| Configure terminal hostname S4112F-TOR1 ! interface mgmt1/1/1 no shutdown no ip address dhcp ip address 192.168.255.1/30 ipv6 address autoconfig ! interface range ethernet1/1/11-1/1/12 description VLTiLink no shutdown no switchport ! interface port-channel10 description CORE.UPLINK no shutdown switchport mode trunk switchport access vlan 1 interface vlan 200 exit interface port-channel10 switchport trunk allowed vlan 200 vlt-port-channel 10 ! interface range ethernet1/1/7-1/1/10 description CORE.UPLINK no shutdown channel-group 10 mode active no switchport flowcontrol receive on flowcontrol transmit off ! vlt-domain 1 backup destination 192.168.255.2 discovery-interface ethernet1/1/11 discovery-interface ethernet1/1/12 vlt-mac 00:00:00:00:00:02 ! interface range ethernet1/1/1-1/1/4 description MGMT.PORTS no shutdown switchport mode trunk switchport access vlan 1 switchport trunk allowed vlan 200 flowcontrol receive on flowcontrol transmit off spanning-tree disable spanning-tree port type edge ! interface range ethernet1/1/13-1/1/14 description STOR.PORTS no shutdown switchport mode trunk switchport access vlan 1 interface vlan 301 interface vlan 302 exit interface range ethernet1/1/13-1/1/14 switchport trunk allowed vlan 301,302 mtu 9216 flowcontrol receive on flowcontrol transmit off spanning-tree disable spanning-tree port type edge ! interface Vlan 200 description MANAGEMENT no shutdown ip address 172.18.100.124/25 vrrp-group 200 virtual-address 172.18.100.126 ! interface Vlan 301 description STORAGE-1 no ip address mtu 9216 no shutdown ! interface Vlan 302 description STORAGE-2 no ip address mtu 9216 no shutdown ! lldp enable ! ip ssh server enable ! write memory | Configure terminal hostname S4112F-TOR2 ! interface mgmt1/1/1 no shutdown no ip address dhcp ip address 192.168.255.2/30 ipv6 address autoconfig ! interface range ethernet1/1/11-1/1/12 description VLTiLink no shutdown no switchport ! interface port-channel10 description CORE.UPLINK no shutdown switchport mode trunk switchport access vlan 1 interface vlan 200 exit interface port-channel10 switchport trunk allowed vlan 200 vlt-port-channel 10 ! interface range ethernet1/1/7-1/1/10 description CORE.UPLINK no shutdown channel-group 10 mode active no switchport flowcontrol receive on flowcontrol transmit off ! vlt-domain 1 backup destination 192.168.255.1 discovery-interface ethernet1/1/11 discovery-interface ethernet1/1/12 vlt-mac 00:00:00:00:00:02 ! interface range ethernet1/1/1-1/1/4 description MGMT.PORTS no shutdown switchport mode trunk switchport access vlan 1 switchport trunk allowed vlan 200 flowcontrol receive on flowcontrol transmit off spanning-tree disable spanning-tree port type edge ! interface range ethernet1/1/13-1/1/14 description STOR.PORTS no shutdown switchport mode trunk switchport access vlan 1 interface vlan 301 interface vlan 302 exit interface range ethernet1/1/13-1/1/14 switchport trunk allowed vlan 301,302 mtu 9216 flowcontrol receive on flowcontrol transmit off spanning-tree disable spanning-tree port type edge ! interface Vlan 200 description MANAGEMENT no shutdown ip address 172.18.100.125/25 vrrp-group 200 virtual-address 172.18.100.126 ! interface Vlan 301 description STORAGE-1 no ip address mtu 9216 no shutdown ! interface Vlan 302 description STORAGE-2 no ip address mtu 9216 no shutdown ! lldp enable ! ip ssh server enable ! write memory |

CONVERGED TOPOLOGY (DCB PFC/ETS)                                                     OS10, 10Gbe/100Gbe

QLOGICTCP RDMA